Router Security Hardening & VLAN Assessment Using GFI LanGuard

Madhuja Singh

A45605222013

B-Tech CSE Sec-B

6th Semester

Table of Contents

### 1. Introduction

1.1 Overview of VLAN Security and the Importance of Vulnerability Scanning .......................... 3
1.2 Introduction to GFI LANguard as a Network Security Scanner ............................................ 4
1.3 Primary Objective .................................................................................................................... 5

### 2. Experiment Setup

2.1 VLAN Configuration and Network Setup .............................................................................. 6
2.2 VLAN Network Setup with Cisco 7200 Router and Layer-2 Switch .................................... 8
2.3 VLAN Configuration on Layer-2 Switch and Cisco 7200 Router ........................................ 9
2.4 Ping Connectivity Test ......................................................................................................... 12

### 3. GFI LANguard Vulnerability Scan (Pre-Implementation)

3.1 Objective of the Scan .......................................................................................................... 13
3.2 Scan Configuration and Target Selection .......................................................................... 13

### 4. Implementing Router Checklist (ISO 27001)

4.1 Introduction .......................................................................................................................... 16
4.2 Security Implementation ...................................................................................................... 17
4.3 Verification of Security Configurations .............................................................................. 22

### 5. Post-Implementation Network Scan

5.1 Windows 10 Scan Results & Vulnerability Assessment ................................................... 23
5.2 Windows 7 Scan Results & Vulnerability Assessment ..................................................... 24
5.3 Cisco Router Scan Results & Vulnerability Assessment ................................................. 25

6. Conclusion ............................................................................................................... 29

## Introduction

### 1.1 Overview of VLAN Security and the Importance of Vulnerability Scanning

A Virtual Local Area Network (VLAN) is a fundamental network segmentation technology that enhances security, optimizes network performance, and reduces broadcast traffic. By logically grouping devices into separate network segments, VLANs prevent unauthorized access between different network segments and limit the spread of security threats such as malware or network attacks.

However, VLANs alone do not provide complete security. Misconfigurations, weak access control policies, and unpatched network devices can create vulnerabilities that attackers might exploit. These vulnerabilities could lead to unauthorized access, data breaches, denial-of-service attacks, or man-in-the-middle exploits.

To mitigate such risks, vulnerability scanning is essential in identifying weaknesses in VLAN configurations. By conducting security audits using specialized tools, administrators can detect open ports, outdated software, misconfigured firewalls, weak encryption mechanisms, and unauthorized devices within VLANs. Regular vulnerability assessments ensure compliance with security standards and help in proactively addressing potential threats before they can be exploited.

### 1.2 Introduction to GFI LanGuard as a Network Security Scanner

GFI LanGuard is an advanced network security scanner and vulnerability assessment tool designed to detect security risks in network environments, including VLAN-based infrastructures. It offers functionalities such as:

- Automatic vulnerability detection by scanning network devices, operating systems, and applications.

- Patch management for updating Windows, macOS, and Linux systems to prevent exploits.

- Network and port scanning to identify exposed services and unauthorized access points.

- Change monitoring to detect unauthorized modifications in system configurations.

In VLAN security auditing, GFI LanGuard plays a critical role by allowing administrators to perform pre- and post-security implementation scans to evaluate network vulnerabilities and measure the effectiveness of security policies.

### 1.3 Primary Objective

The primary goal of this experiment is to assess VLAN security using GFI LanGuard, implement the ISO router security checklist, and compare network security status before and after applying security measures. The steps include:

- Conducting an initial vulnerability scan of VLANs to detect existing security gaps.

- Implementing the ISO Router Security Checklist, including measures such as firewall rules, port security, ACLs (Access Control Lists), disabling unused services, and enabling encryption protocols.

- Performing a second scan using GFI LanGuard to compare the security posture after implementing the checklist.

Experiment Setup

### 2.1 VLAN Configuration and Network Setup

To effectively conduct the VLAN security assessment, a layered network infrastructure was implemented, consisting of:

- A Cisco 7200 Router (C7200), which is widely used for enterprise networking and high-performance routing. It supports multiple interfaces, advanced security features, and VLAN routing (Inter-VLAN Routing), making it ideal for this experiment.

- A Layer 2 Switch, responsible for VLAN segmentation. Layer 2 switches operate at the Data Link Layer (Layer 2) and forward traffic based on MAC addresses, ensuring efficient communication between devices within the same VLAN.

The network was designed with two VLANs, each assigned to different subnets:

- VLAN 10: Assigned IP range 192.168.3.0/24

- VLAN 20: Assigned IP range 192.168.4.0/24

Each VLAN was configured on the Layer 2 switch, with trunking enabled to allow communication with the Cisco 7200 router for inter-VLAN routing. The router acted as the default gateway for each VLAN, facilitating traffic flow between VLANs while enforcing security policies.

Figure 1

Two devices were connected to this VLAN setup:

- Windows 10 machine (assigned to VLAN 10)

- Windows 7 machine (assigned to VLAN 20)

2.2 VLAN Network Setup with Cisco 7200 Router and Layer- 2 Switch

The following diagram/screenshot represents the VLAN setup implemented for this experiment. It illustrates the network topology, including the Cisco 7200 router, Layer 2 switch, and VLAN assignments. This setup was used to test network segmentation and security policies before conducting a vulnerability scan with GFI LanGuard.

Figure 2

As shown in the above setup, the router-on-a-stick (ROAS) configuration enables inter-VLAN communication via sub interfaces on the Cisco 7200 router. VLANs are configured on the Layer 2 switch, and trunking is used for VLAN tagging.

2.3 VLAN Configuration on Layer 2 Switch and Cisco 7200- Router

Switch Configuration:

Figure 3

Router Configuration:

Figure 4

The Windows 10 and Windows 7 machines were assigned static IP addresses based on their respective VLANs. The configurations were as follows:

- Windows 10 (VLAN 10) Configuration:

  - IP Address: 192.168.3.10

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.3.1

- Windows 7 (VLAN 20) Configuration:

  - IP Address: 192.168.4.10

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.4.1

Device IP Configuration (PC1- WINDOWS 10)                                                  Figure 6

Device IP Configuration (PC2- WINDOWS 7)

Figure 7

These configurations ensured that each device was assigned an IP address within its designated VLAN subnet. The default gateway was set to the respective VLAN sub interface on the Cisco 7200 router, enabling inter-VLAN communication when required.

With the IP configurations set, the next step was to verify connectivity within the VLANs using ping tests before proceeding to the VLAN security scan with GFI LanGuard.

2.4 Ping Connectivity Test

After configuring the VLANs and assigning IP addresses, connectivity tests were conducted using the ping command to verify communication between devices.

Figure 8- Ping from PC1 To PC2

Figure-9 Ping from PC2 To PC1

These results were recorded as a baseline before proceeding with the GFI LanGuard security scan, which would assess network vulnerabilities and compliance with the ISO Router Security Checklist.

GFI LanGuard Vulnerability Scan (Pre-Implementation)

#### 3.1 Objective of the Scan

The primary objective of the initial scan was to assess the security posture of the VLAN setup before implementing the ISO Router Security Checklist. The scan aimed to:

- Identify vulnerabilities within the VLAN configuration, including unpatched systems, open ports, and misconfigurations.

- Detect potential security risks on the router, switch, and connected devices.

- Establish a security baseline for comparison after implementing security enhancements.

#### 3.2 Scan Configuration and Target Selection

To conduct a comprehensive vulnerability assessment, the following configuration settings were applied in GFI LanGuard:

- Target Selection: The scan was performed on the entire VLAN network, including the router, switches, and connected devices. The specific IP addresses scanned were:

  - VLAN 10 Subnet: 192.168.3.0/24

  - VLAN 20 Subnet: 192.168.4.0/24

  - Router IPs: 192.168.3.1, 192.168.4.1

- Scan Type: A full vulnerability scan was selected to detect security flaws, missing updates, and network vulnerabilities.

- Execution: The scan was initiated in GFI LanGuard and monitored until completion.

The above image shows the specific IP addresses selected for the GFI LanGuard vulnerability scan. Instead of scanning the entire subnet, the scan was focused on key network devices to optimize efficiency and reduce scan time:

- 192.168.3.10 (Windows 10 PC in VLAN 10)

- 192.168.4.10 (Windows 7 PC in VLAN 20)

- 192.168.3.1 (Router Interface for VLAN 10)

- 192.168.4.1 (Router Interface for VLAN 20)

Figure 11: GFI LanGuard Scanning Process

The above image captures the real-time scanning process performed using GFI LanGuard. The scan was conducted to identify vulnerabilities, missing patches, and security risks across the selected VLAN devices.

During the scan, GFI LanGuard performed the following actions:

- Network Discovery: Identified active devices within the specified IP range.

- Vulnerability Assessment: Checked for outdated software, misconfigurations, and potential security threats.

- Patch Management: Evaluated whether critical security updates were missing.

This initial scan serves as a baseline assessment before implementing ISO Router Security Checklist measures. The results from this scan will later be compared with a post-implementation scan to analyze security improvements.

Implementing Router Checklist ISO 27001

#### 4.1 Introduction

ISO 27001 is an internationally recognized standard for information security management, designed to establish, implement, maintain, and continually improve an organization's information security posture. Routers play a critical role in network security, acting as the first line of defense against potential cyber threats. Implementing ISO 27001 best practices on network devices, such as routers, helps mitigate vulnerabilities, prevent unauthorized access, and ensure data integrity by enforcing strict security controls.

In this experiment, applying the ISO 27001 security checklist to the Cisco router strengthens VLAN security by reducing exposure to potential attacks. Before implementation, a GFI LanGuard scan was conducted to assess existing vulnerabilities in the network. After applying security policies—such as enabling SSH, disabling Telnet, implementing access control lists (ACLs), and disabling unused services—another scan was performed to evaluate the effectiveness of these measures. The comparison of pre- and post-implementation scan results highlights the improvements in network security, demonstrating how ISO 27001 guidelines enhance router security and protect VLAN communication from unauthorized access and potential threats.

1.Disabling Unused Services and Interfaces

Disabling unused services and interfaces minimizes potential entry points for attackers, reducing the risk of unauthorized access or exploitation. By shutting down inactive interfaces and disabling unnecessary services such as HTTP and Telnet, the router's security posture is strengthened. This ensures that only essential network functions remain active, aligning with best practices for securing network infrastructure.

Commands Executed:

Figure 12

### 2. Securing Router Access with SSH

To enhance network security and comply with ISO 27001 guidelines, SSH was enabled on the Cisco router while Telnet was disabled. This ensures encrypted remote access, preventing unauthorized logins and securing administrative sessions.

The following configurations were implemented:

- Console Access Protection: A password was set for direct console access, ensuring only authorized users can manage the router locally.

- VTY Line Security: A password was assigned to virtual terminal (VTY) lines, and SSH was enforced as the only remote access protocol, preventing unencrypted Telnet connections.

Commands Executed:

Figure 13

### 3. Enabling Secure Remote Access via SSH

To enhance security, SSH was enabled on the router, replacing Telnet. This ensures encrypted communication and prevents unauthorized access. The following configurations were implemented:

- Generated RSA keys (2048-bit) to enable SSH.

- Configured VTY lines to allow only SSH and enforce local authentication.

- Created a privileged admin account with an encrypted password.

- Set SSH version to 2 for enhanced security.

Commands Executed:

Figure 14

This configuration secures remote access by encrypting login sessions and blocking Telnet.

4. Restricting Remote Access with ACLs

To enhance security, an Access Control List (ACL) was applied to restrict SSH access to specific VLANs while blocking all other traffic. This ensures that only authorized subnets can establish SSH connections.

#### Implemented Configurations:

- Allowed SSH (port 22) from VLAN 10 (192.168.3.0/24) and VLAN 20 (192.168.4.0/24).

- Denied all other IP traffic for security.

- Applied ACL 100 to VTY lines to enforce access restrictions.

Commands Executed:

Figure 15

This configuration ensures that only devices from VLAN 10 and VLAN 20 can remotely access the router via SSH while blocking all unauthorized attempts.

5. Implementing Logging for Security Monitoring

To enhance network security and facilitate troubleshooting, logging was configured on the router. These settings ensure that important system events are recorded for analysis and incident response.

#### Configured Logging Settings:

- Buffered Logging: Stores log messages in the router’s memory (buffer size set to 4096).

- Console Logging: Displays informational messages on the console for real-time monitoring.

- Monitor Logging: Sends informational logs to remote monitoring sessions.

- Trap Logging: Sends warning-level logs to an external syslog server if configured.

Commands Executed:

Figure 16

6. Configuring Time Zone and System Clock

To ensure accurate timestamping of logs and synchronization with security policies, the router’s time zone was configured. Proper time settings are crucial for log analysis, troubleshooting, and compliance with security standards like ISO 27001.

#### Configured Settings:

- Clock Time zone: Set to UTC +0 to maintain consistency across network logs.

- System Clock Update: The router's internal clock was updated accordingly.

Commands Executed:

Figure 17

### 4.3 Verification of Security Configurations

After implementing the security policies, the router was reloaded to ensure the changes took effect. Upon restart, the system prompted for authentication, confirming that the configurations were successfully applied.

#### Verification Steps:

- Router Reloaded: A restart was performed to validate the persistence of the applied security settings.

- Authentication Prompt: The router requested login credentials, ensuring that password protection was enforced.

- Configuration Check: The applied security measures, including SSH access control, logging, and access lists, remained intact after reboot.

Figure 18

A screenshot of the authentication prompt has been attached as evidence of successful implementation. This verification confirms that the ISO 27001 security policies were effectively enforced on the router.

### Post-Implementation Network Scan

5.1 Windows 10 Scan Results & Vulnerability Assessment

### Figure 19- Windows 10 Scan Results

After implementing the ISO 27001 security policies, a comprehensive scan was conducted using GFI LanGuard on the Windows 10 machine. The scan successfully detected and categorized vulnerabilities as follows:

- High-Risk Vulnerabilities: 3

- Medium-Risk Vulnerabilities: 0

- Low-Risk Vulnerabilities: 5

- Potential Vulnerabilities: 1

These results demonstrate GFI LanGuard’s effectiveness in identifying security gaps within endpoint devices by performing in-depth vulnerability assessments. The tool successfully scanned the Windows 10 machine, detecting critical risks that require attention, along with lower-priority issues that contribute to the overall security posture.

Unlike the router, where security improvements were clearly observed, the Windows 10 scan highlights existing system-level vulnerabilities that were not directly influenced by the network security changes. However, this further reinforces the importance of both network and endpoint security measures in a layered security approach.

5.2 Windows 7 Scan Results & Vulnerability Assessment

Figure 20

During the scanning process, GFI LanGuard detected only three open ports on the Windows 7 machine but did not generate detailed vulnerability results. This limited scan response can be attributed to several factors, primarily related to the age and security architecture of Windows 7:

- Outdated Operating System: Since Windows 7 has reached end-of-life support, its security mechanisms and system responses differ from modern operating systems like Windows 10. Many advanced vulnerability scanning techniques are designed for actively maintained operating systems, which may have contributed to fewer scan results.

- Legacy System Security Differences: Older operating systems often have different default security configurations, firewall settings, or missing newer security event logging features that modern scanning tools rely on for deeper vulnerability detection.

Despite the limited findings, the detection of three open ports on Windows 7 still provides useful insights. It highlights the importance of securing legacy systems, as they can still pose potential risks within a network. The scan results reinforce the idea that outdated operating systems require additional security measures, such as network segmentation, firewall configurations, and endpoint hardening, to mitigate potential risks effectively.

5.3 Cisco Routers Scan Results & Vulnerability Assessment

1.Device Identification Improvement:

Figure 21

- Before implementing security policies, some devices were identified as "Probably Unix."

Figure 22

After the changes, GFI LanGuard correctly identified network devices as "Cisco," indicating improved recognition and more accurate scanning.

### 2. Reduction in Vulnerability Level:

- Before implementing the security policies, the overall vulnerability level of the network was rated as Medium due to exposed services, lack of encryption, and potential unauthorized access points.

- After applying the ISO 27001 security measures, including SSH enforcement, access control lists, and logging configurations, the vulnerability level was reduced to Low in the post-implementation scan.

- This reduction indicates that the network is now more secure, with minimized attack surfaces and better compliance with security best practices.

Figure 23- Before Implementing Router Security

Figure 24- After Implementing Router Security

A comparison of the vulnerability levels before and after implementation has been attached to demonstrate the effectiveness of these security enhancements.

### 3.Open Ports Analysis – Pre and Post Security Implementation:

One of the key observations from the vulnerability scans was the change in the number of open ports before and after implementing the ISO 27001 security policies on the router.

#### Before Security Implementation:

- The scan detected 12 open ports on the router.

- These included common ports such as SSH (22), HTTP (80), HTTPS (443), and Telnet (23) along with other service-related ports.

- Having multiple open ports increases the attack surface, making the device more vulnerable to unauthorized access or exploitation.

#### After Security Implementation:

- The number of open ports reduced from 12 to 10.

- The decrease in open ports indicates that some unnecessary or insecure services were successfully disabled as part of the security hardening process.

- This reduction aligns with best security practices, as minimizing open ports limits potential entry points for attackers.

Figure 25- Before Implementing Security Number of Ports:12

Figure 26- After Implementing Security Number of Ports Decreased to 10

The results from the three router scans demonstrate the effectiveness of implementing the ISO 27001 security checklist. This highlights the importance of proactive security implementations in safeguarding network infrastructure against potential threats.

Conclusion

This experiment successfully demonstrated the impact of implementing security measures on a Cisco router within a VLAN-configured network. Using GFI LanGuard, vulnerability assessments were conducted before and after applying the ISO 27001 Router Security Checklist, highlighting key security improvements.

The initial scan revealed that network devices, including the router, were not optimally secured, with medium-level vulnerabilities and multiple open ports. After implementing the security configurations—such as enabling SSH, restricting Telnet, applying access control lists (ACLs), disabling unused services, and configuring logging—the scan results showed significant improvements. The router’s device identification accuracy improved, the vulnerability level was reduced, and the number of open ports decreased from 12 to 10.

For endpoint devices, Windows 10 was effectively scanned, detecting 3 high, 5 low, and 1 potential vulnerability, showcasing GFI LanGuard’s capability in identifying security risks. However, Windows 7 did not generate a complete scan report, likely due to its outdated architecture and limited security response mechanisms. It only reported three open ports, indicating restricted detection capabilities.

Overall, the experiment highlighted the importance of securing routers in VLAN environments, as they serve as critical control points for inter-VLAN communication and overall network security. The implementation of ISO 27001 guidelines significantly enhanced router security, reducing vulnerabilities and strengthening the network’s defense against potential threats.